# User Manual for Janelia Ethoscope

# Author: Salma Elmalaki

Original User Manual is by  Gilestro lab:

https://qgeissmann.gitbooks.io/ethoscope-manual/using-ethoscopes/starting-your-first-experiment.html

# Ethoscope main page

# Use Google Chrome

# Use http://0.0.0.0:8000 to connect to the Ethoscope server

# The available devices will be listed, click on the required device to set its testing parameters

# A new tab will be opened when you click on one of the devices

# Press “Start tracking”

# Choose the interactor

# Default Stimulator: No stimulation is applied, just tracking.

# JaneliaAdaptiveSleepDepStimulator:

# Moto-stimulus is applied in an adaptive way, no opto stimulus

# JaneliaShakerSleepDepStimulator:

# Moto-stimulus as an oscillation of the tube, no opto stimulus, not adaptive

# JaneliaOptoMotorAdaptiveSleepDepStimulator:

# Same as JaneliaAdaptiveSleepDepStimulator but with opto-stimulus

# JaneliaOptoShakerSleepDepStimulator:

# Same as JaneliaShakerSleepDepStimulator but with opto-stimulus

# Select the parameters

# The parameters are loaded with default values as shown in the figure.

# You can also specify the date and time range for which the stimulus will be applied. By default (empty), the stimulus will be applied as soon as the experiment starts.

# Tracking page

# The tracking information is opened for the specified device.

# The tracking information is saved in /ethoscope_results/ but you can directly downloaded it from this page by clicking on it (the file is available after 5 min from the start of the experiment)

# Management page

# Click the Management tab and you can have access to all the databases collected by the server over several experiments and for multiple devices.

# Update page

# Select the Update System tab and you can update the server (node), and the devices with the github code if there are any changes in master repo.

# The red circle beside the device indicates that it needs to be updated with the latest repo version.

# If it is updated, the circle will be green.

# To update the devices, they should not be running (no active tracking) is taking place while you do the update. You will receive an update error if you attempt to update a running device.

# Troubleshooting

# To restart the server:

# On the terminal:

# sudo systemctl restart ethoscope_node.service

# sudo systemctl restart ethoscope_update_node.service

# sudo systemctl restart ethoscope_backup.service

# To restart any device:

# On the terminal:

# ssh pi@192.168.123.3  (or whatever ip of the device you want to restart)

# Pass: e001@hhmi

# Sudo systemctl restart ethoscope_device.service

# Sudo systemctl restart ethoscope_update.service

# To view the service log messages, for example, “ethoscope_node.service”

# sudo journalctl -u ethoscope_node.service -e   on the server machine

# Adaptive Stimulus

# How to set the parameters in the adaptive stimulus.

# Acceleration parameters are also set the same way.